# Lab 7 Report

In this lab, we need to find another way to increase clock speed. To do this, we will be “pipelining” the design by adding registers in-between gates of the circuit to split the data path into various segments. Each segment will take less time than the overall circuit so the overall throughput will be higher.

# 1 Observation & correction:

## Before fix:

module ALT_MULTADD(iCLK, iRST_N, iSEL,  iA0, iA1, iB0, iB1, oR);

input iCLK, iRST_N, iSEL, iA0, iA1, iB0, iB1;

output oR;

wire iCLK, iRST_N, iSEL;

wire [7:0] iA0, iA1, iB0, iB1;

reg [7:0] A0, A1, B0, B1;

reg [16:0] oR;

always @ (posedge iCLK, negedge iRST_N)begin

A0 <= iA0;

A1 <= iA1;

B0 <= iB0;

B1 <= iB1;

if(iRST_N)begin

if(iSEL) begin

oR <= A0 * B0 + A1 * B1;

end

else begin

oR <= A0 * B0 + A1 * B1 + A0 * A1 * B0 * B1;

end

end

else begin

oR <= 0;

end

end

endmodule

### Simulation:

# Test control

#     iCLK_t   iRST_N_t     iSEL_t      iA0_t      iA1_t      iB0_t      iB1_t  oRESULT_t

#          0          0          0          0          0          0          0          0

#          1          1          0          0          0          0          0          0

#          0          1          0          3          3          3          3          0

#          1          1          0          3          3          3          3          0

#          0          1          1          1          1          1          1          0

#          1          1          1          1          1          1          1         18

#          0          1          1          1          1          1          1         18

#          1          1          1          1          1          1          1          2

#          0          1          1          1          1          1          1          2

#          1          1          1          1          1          1          1          2

#          0          1          1          1          1          1          1          2

#          1          1          1          1          1          1          1          2

#          0          1          1          1          1          1          1          2

#          1          1          1          1          1          1          1          2

#          0          1          0          2          2          2          2          2

#          1          1          0          2          2          2          2          3

#          0          1          0          2          2          2          2          3

This is not correct, we need the first output from oRESULT to correspond to the input from the previous clock cycle.

## After fix:

module ALT_MULTADD(iCLK, iRST_N, iSEL,  iA0, iA1, iB0, iB1, oR);

input iCLK, iRST_N, iSEL, iA0, iA1, iB0, iB1;

output oR;

wire iCLK, iRST_N, iSEL;

wire [7:0] iA0, iA1, iB0, iB1;

reg SEL;

reg [7:0] A0, A1, B0, B1;

reg [16:0] oR;

always @ (posedge iCLK, negedge iRST_N)begin

A0 <= iA0;

A1 <= iA1;

B0 <= iB0;

B1 <= iB1;

SEL <= iSEL;

if(iRST_N)begin

if(SEL) begin

oR <= A0 * B0 + A1 * B1;

end

else begin

oR <= A0 * B0 + A1 * B1 + A0 * A1 * B0 * B1;

end

end

else begin

oR <= 0;

end

end

endmodule

Simulation:

`timescale 10ns/1ns

module ALT_MULTADD_TB ();

reg iCLK_t, iRST_N_t, iSEL_t;

reg [7:0] iA0_t, iA1_t, iB0_t, iB1_t;

wire [16:0] oRESULT_t;

ALT_MULTADD X(iCLK_t, iRST_N_t, iSEL_t,  iA0_t, iA1_t, iB0_t, iB1_t, oRESULT_t);

initial $display ("Test control");

initial $display ("%10s %10s %10s %10s %10s %10s %10s %10s",

"iCLK_t", "iRST_N_t", "iSEL_t", "iA0_t", "iA1_t",

"iB0_t", "iB1_t", "oRESULT_t");

initial $monitor ("%10b %10b %10b %10d %10d %10d %10d %10d",

iCLK_t, iRST_N_t, iSEL_t,  iA0_t, iA1_t, iB0_t, iB1_t, oRESULT_t);

initial begin

iCLK_t = 0;

iRST_N_t = 0;

iSEL_t = 0;

iA0_t = 0;

iA1_t = 0;

iB0_t = 0;

iB1_t = 0;

#1

iRST_N_t = 1;

#1

iA0_t = 3;

iA1_t = 3;

iB0_t = 3;

iB1_t = 3;

#2

iSEL_t = 1;

iA0_t = 1;

iA1_t = 1;

iB0_t = 1;

iB1_t = 1;

#4

$stop;

end

always #1 iCLK_t = ~iCLK_t;

endmodule

Good, the transition of oResult from 99 to 2 shows that the new design is working properly.

# Test control

#     iCLK_t   iRST_N_t     iSEL_t      iA0_t      iA1_t      iB0_t      iB1_t  oRESULT_t

#          0          0          0          0          0          0          0          x

#          1          1          0          0          0          0          0          x

#          0          1          0          3          3          3          3          x

#          1          1          0          3          3          3          3          0

#          0          1          1          1          1          1          1          0

#          1          1          1          1          1          1          1         99

#          0          1          1          1          1          1          1         99

#          1          1          1          1          1          1          1          2

#          0          1          1          1          1          1          1          2

#          1          1          1          1          1          1          1          2

#          0          1          1          1          1          1          1          2

#          1          1          1          1          1          1          1          2

#          0          1          1          1          1          1          1          2

#          1          1          1          1          1          1          1          2

#          0          1          0          2          2          2          2          2

#          1          1          0          2          2          2          2          2

#          0          1          0          2          2          2          2          2

#          1          1          0          2          2          2          2         24

#          0          1          0          2          2          2          2         24

#          1          1          0          2          2          2          2         24

#          0          1          0          2          2          2          2         24

#          1          1          0          2          2          2          2         24

#          0          1          0          2          2          2          2         24

# 2 Pipelining

## Schematic:

## & Code:

module ALT_MULTADD_pipe(iCLK, iRST_N, iSEL,  iA0, iA1, iB0, iB1, oR);

input iCLK, iRST_N, iSEL, iA0, iA1, iB0, iB1;

output oR;

wire iCLK, iRST_N, iSEL;

wire [7:0] iA0, iA1, iB0, iB1;

reg SEL, SEL1;

reg [7:0] A0, A1, B0, B1, M0, M1;

reg [16:0] oR;

always @ (posedge iCLK)begin

A0 <= iA0;

A1 <= iA1;

B0 <= iB0;

B1 <= iB1;

M0 <= A0 * B0;

M1 <= A1 * B1;

SEL1 <= iSEL;

SEL <= SEL1;

if(iRST_N)begin

if(SEL) begin

oR <= M0 + M1;

end

else begin

oR <= M0 + M1 + M0 * M1;

end

end

else begin

oR <= 0;

end

end

endmodule

# 3 Pipelined Design Waveform:

As you can see, the pipelining causes an extra clock cycle before output register outputs but throughput and functioning is still the same.

# 4 Synthesis:

We synthesized both the circuits from above and compared differences between un-piped and piped designs.

## Design constraints:

After tuning the clock period, the lowest we were able to achieve is .93 ns.

## Synthesized Schematic:

We see the registers located in-between the first logic computation stage and the second. This is what we want to see.

## Area report comparison:

We will compare the differences in area consumption between piped and non-piped designs.

### Non-Pipelined:

Instance           Cells  Cell Area  Net Area  Total Area    Wireload

-------------------------------------------------------------------------------------

ALT_MULTADD                   1399       4245         0        4245 ZeroWireload (S)

csa_tree_add_20_28           362       1125         0        1125 ZeroWireload (S)

csa_tree_add_23_38_groupi    385       1113         0        1113 ZeroWireload (S)

csa_tree_mul_23_48_groupi    327        856         0         856 ZeroWireload (S)

mul_23_43                    222        604         0         604 ZeroWireload (S)

final_adder_add_20_28         34        152         0         152 ZeroWireload (S)

### Pipelined:

Instance       Cells  Cell Area  Net Area  Total Area    Wireload

----------------------------------------------------------------------------

ALT_MULTADD_pipe     2550       5813         0        5813 ZeroWireload (S)

RC_CG_HIER_INST0      1          6         0           6 ZeroWireload (S)

sharing_mux60         1          2         0           2 ZeroWireload (S)

The area increased from 4245 to 5813. This is expected because of the inserted registers which require extra area.

### Retime enabled:

Instance                     Cells  Cell Area  Net Area  Total Area    Wireload

---------------------------------------------------------------------------------------------------------

ALT_MULTADD_pipe                                  1370       3820         0        3820 ZeroWireload (S)

csa_tree_add_25_28                               605       1517         0        1517 ZeroWireload (S)

mul_19_14                                        262        675         0         675 ZeroWireload (S)

mul_20_14                                        249        669         0         669 ZeroWireload (S)

final_adder_add_25_28_Y_final_adder_add_22_22    130        384         0         384 ZeroWireload (S)

sharing_mux                                       25         64         0          64 ZeroWireload (S)

sharing_mux59                                     25         57         0          57 ZeroWireload (S)

RC_CG_HIER_INST0                                   1          6         0           6 ZeroWireload (S)

sharing_mux60                                      1          2         0           2 ZeroWireload (S)

We can see that the types of cells synthesized is different from when retime was not enabled.

## Timing Comparison:

Here, we will compare the timing of the non-pipelined vs the pipelined logic.

Here are the pipelined timing reports:

### Pipelined:

Pin                 Type     Fanout Load Slew Delay Arrival

(fF) (ps)  (ps)   (ps)

--------------------------------------------------------------------------

(clock iCLK)                   launch                                 0 R

M1_reg[3]/CP                                            0             0 R

M1_reg[3]/Q                    DFQD4          5  9.4   33  +127     127 F

g11957/I                                                     +0     127

g11957/ZN                      CKND4          6  9.2   26   +25     152 R

csa_tree_add_25_28_g13816/I                                  +0     152

csa_tree_add_25_28_g13816/ZN   CKND2          5  8.7   33   +28     180 F

csa_tree_add_25_28_g8295/A2                                  +0     180

csa_tree_add_25_28_g8295/ZN    CKND2D2        1  3.4   34   +31     211 R

csa_tree_add_25_28_g8105/A2                                  +0     211

csa_tree_add_25_28_g8105/ZN    ND2D3          6  6.8   34   +34     245 F

csa_tree_add_25_28_g7643/B1                                  +0     245

csa_tree_add_25_28_g7643/ZN    MOAI22D1       2  2.3   43   +64     309 F

csa_tree_add_25_28_g7515/A1                                  +0     309

csa_tree_add_25_28_g7515/ZN    CKND2D1        2  3.3   54   +42     351 R

g13591/A2                                                    +0     351

g13591/ZN                      CKND2D2        2  2.3   23   +30     380 F

g319/A1                                                      +0     380

g319/Z                         OA21D1         2  2.8   31   +62     443 F

g10928/A1                                                    +0     443

g10928/ZN                      NR2XD1         2  2.1   32   +27     470 R

g10931/A1                                                    +0     470

g10931/ZN                      MOAI22D1       1  1.1   32   +26     496 F

g12279/A1                                                    +0     496

g12279/ZN                      CKND2D1        1  1.2   32   +27     522 R

g12278/B                                                     +0     522

g12278/ZN                      IOA21D1        2  2.8   43   +35     557 F

g13149/A2                                                    +0     557

g13149/ZN                      NR2XD1         3  3.8   46   +45     602 R

g11685/A1                                                    +0     602

g11685/ZN                      NR2XD1         1  2.1   25   +26     628 F

g11684/A1                                                    +0     628

g11684/ZN                      NR2D2          3  3.3   46   +34     663 R

g12706/A1                                                    +0     663

g12706/ZN                      NR3D1          1  1.7   31   +26     689 F

g13640/A1                                                    +0     689

g13640/ZN                      NR2XD1         1  4.0   48   +36     724 R

g13249/A1                                                    +0     724

g13249/ZN                      CKND2D4        8  8.8   33   +32     757 F

g11694/I                                                     +0     757

g11694/ZN                      CKND0          1  1.2   32   +29     786 R

g35/A1                                                       +0     786

g35/ZN                         NR2D1          2  2.4   25   +22     808 F

g12966/A1                                                    +0     808

g12966/ZN                      OAI21D1        1  1.2   47   +35     843 R

g845/A1                                                      +0     843

g845/ZN                        ND3D1          1  1.2   46   +41     884 F

g844/A1                                                      +0     884

g844/ZN                        ND2D1          1  1.1   26   +26     909 R

oRESULT_reg[11]/D              DFQD1                         +0     909

oRESULT_reg[11]/CP             setup                    0   +31     940 R

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

(clock iCLK)                   capture                              940 R

--------------------------------------------------------------------------

Cost Group   : 'iCLK' (path_group 'iCLK')

Timing slack :       0ps

Start-point  : M1_reg[3]/CP

End-point    : oRESULT_reg[11]/D

### Non- Pipelined:

Pin            Type      Fanout Load Slew Delay Arrival

(fF) (ps)  (ps)   (ps)

-----------------------------------------------------------------

(clock iCLK)         launch                                  0 R

A0_reg[5]/CP                                   0             0 R

A0_reg[5]/Q          DFQD1          16 19.0  169  +183     183 R

mul_23_43/A[5]

g3067/A2                                          +0     183

g3067/ZN           ND2D1           2  2.2   62   +59     242 F

g2991/A2                                          +0     242

g2991/ZN           XNR2D1          2  3.3   44  +112     354 F

g3097/A1                                          +0     354

g3097/Z            XOR3D1          2  3.3   48  +190     544 R

g3092/A1                                          +0     544

g3092/ZN           XNR3D1          2  2.3   40  +193     737 R

g2881/A1                                          +0     737

g2881/ZN           NR2D1           2  2.2   24   +24     761 F

g2856/B                                           +0     761

g2856/ZN           AOI21D1         3  3.6   94   +71     832 R

g2855/I                                           +0     832

g2855/ZN           INVD1           1  1.2   28   +27     859 F

g2842/A1                                          +0     859

g2842/Z            AO21D1          2  2.8   32   +62     921 F

g2831/A1                                          +0     921

g2831/ZN           XNR2D1          9 10.8   80  +111    1032 F

mul_23_43/Z[7]

csa_tree_mul_23_48_groupi/in_0[7]

g4897/B1                                          +0    1032

g4897/ZN           MOAI22D1        2  2.2   88   +75    1108 F

g4859/B1                                          +0    1108

g4859/ZN           AOI22D1         2  3.3  104   +90    1198 R

g4981/A1                                          +0    1198

g4981/ZN           XNR3D1          2  2.1   39  +203    1401 R

g4681/A1                                          +0    1401

g4681/ZN           CKND2D1         2  2.1   43   +32    1433 F

g4669/A1                                          +0    1433

g4669/ZN           INR2D1          2  2.4   28   +58    1490 F

g4654/A1                                          +0    1490

g4654/ZN           MOAI22D1        9 10.7  213  +127    1618 R

csa_tree_mul_23_48_groupi/out_0[7]

csa_tree_add_23_38_groupi/in_0[7]

g6496/B1                                          +0    1618

g6496/ZN           MAOI22D1        2  1.5   78   +94    1712 R

g6404/B2                                          +0    1712

g6404/Z            OA22D0          2  3.3   74  +112    1824 R

g6587/A1                                          +0    1824

g6587/ZN           XNR3D1          2  3.7   51  +207    2031 R

g6578/A2                                          +0    2031

g6578/ZN           XNR3D1          2  3.2   47  +201    2232 R

g6247/B                                           +0    2232

g6247/ZN           MAOI222D1       2  2.8  113   +93    2325 F

g6564/A2                                          +0    2325

g6564/ZN           XNR3D1          2  2.3   40  +212    2536 R

g6189/A1                                          +0    2536

g6189/ZN           NR2D1           2  2.1   40   +23    2560 F

g6124/B                                           +0    2560

g6124/Z            AO21D1          3  3.6   36   +85    2645 F

g6122/A2                                          +0    2645

g6122/ZN           AOI21D1         2  2.4   73   +53    2697 R

g6121/B1                                          +0    2697

g6121/ZN           MOAI22D1        1  0.8   44   +65    2763 R

csa_tree_add_23_38_groupi/out_0[16]

g491/A1                                             +0    2763

g491/Z               AO22D0          1  1.1   47   +76    2838 R

oRESULT_reg[16]/D    DFQD1                          +0    2838

oRESULT_reg[16]/CP   setup                     0   +35    2874 R

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

(clock iCLK)         capture                              3000 R

-----------------------------------------------------------------

Cost Group   : 'iCLK' (path_group 'iCLK')

Timing slack :     126ps

Start-point  : A0_reg[5]/CP

End-point    : oRESULT_reg[16]/D

The setup rise time is much shorter for the pipelined version as you can see it is 940ps for the pipelined and 3000 ps for the non-pipelined version.

### Retime Enabled:

============================================================

Generated by:           Encounter(R) RTL Compiler v12.10-s012_1

Generated on:           Oct 18 2015  06:50:41 pm

Module:                 ALT_MULTADD_pipe

Technology library:     tcbn65gpluswc 121

Operating conditions:   WCCOM (balanced_tree)

Wireload mode:          segmented

Area mode:              timing library

============================================================

Pin              Type      Fanout Load Slew Delay Arrival

(fF) (ps)  (ps)   (ps)

--------------------------------------------------------------------

(clock iCLK)            launch                                  0 R

retime_s2_31_reg/CP                               0             0 R

retime_s2_31_reg/Q      DFQD2          12 16.4   55  +155     155 F

csa_tree_add_25_28/in_3[9]

g20821/I                                             +0     155

g20821/ZN             INVD4          16 18.0   47   +41     196 R

g20970/A2                                            +0     196

g20970/ZN             IOA21D1         1  3.6   50   +81     277 R

g20531/A2                                            +0     277

g20531/ZN             NR2XD2          8  8.2   36   +38     315 F

g20966/B1                                            +0     315

g20966/Z              AO22D1          2  2.3   33   +87     402 F

g20245/A2                                            +0     402

g20245/ZN             NR2D1           1  2.2   59   +47     449 R

g20223/B                                             +0     449

g20223/ZN             IAO21D2         1  2.1   28   +21     470 F

g20997/CI                                            +0     470

g20997/S              FICOND2         4  4.7   42   +96     567 R

g20121/B1                                            +0     567

g20121/ZN             MAOI22D1        2  2.4   73   +78     645 R

g20083/B1                                            +0     645

g20083/ZN             MOAI22D1        3  4.5  110   +82     727 R

g20059/A                                             +0     727

g20059/ZN             MAOI222D1       1  1.2   84   +72     800 F

csa_tree_add_25_28/out_0[13]

sharing_mux59/in_0[13]

g691/A1                                              +0     800

g691/Z                AO21D1          2  2.1   29   +73     872 F

sharing_mux59/z[13]

final_adder_add_25_28_Y_final_adder_add_22_22/B[13]

g1837/A2                                             +0     872

g1837/ZN              NR2XD0          1  1.1   38   +34     907 R

retime_s3_13_reg/D    DFQD1                          +0     907

retime_s3_13_reg/CP   setup                     0   +33     940 R

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

(clock iCLK)            capture                               940 R

--------------------------------------------------------------------

Cost Group   : 'iCLK' (path_group 'iCLK')

Timing slack :       0ps

Start-point  : retime_s2_31_reg/CP

End-point    : final_adder_add_25_28_Y_final_adder_add_22_22/retime_s3_13_reg/D

With retime enabled, we see don’t see much change other than the types and number of gates used. The slack time is same as the pipelined without retime enabled.

## Power Comparison:

We will compare the power consumption of the non-piplined vs the pipelined logic.

### Non-pipelined:

Instance          Cells Power(nW)  Power(nW)   Power(nW)

--------------------------------------------------------------------

ALT_MULTADD                  1399 36145.955 2006105.916 2042251.871

csa_tree_add_20_28          362  9827.651  396998.770  406826.422

csa_tree_add_23_38_groupi   385  9390.185  661773.301  671163.486

csa_tree_mul_23_48_groupi   327  7201.283  460632.382  467833.666

mul_23_43                   222  5362.549  196315.693  201678.242

final_adder_add_20_28        34  1510.783   76452.623   77963.406

### Pipelined:

Leakage    Dynamic      Total

Instance      Cells Power(nW)  Power(nW)   Power(nW)

-----------------------------------------------------------

ALT_MULTADD_pipe    2550 56930.755 4937303.673 4994234.428

RC_CG_HIER_INST0     1    41.184   36125.103   36166.288

sharing_mux60        1    11.875     567.993     579.868

### Pipelined Retime:

Leakage    Dynamic      Total

Instance          Cells Power(nW)  Power(nW)   Power(nW)

--------------------------------------------------------------------

ALT_MULTADD_pipe             1370 31988.559 3570951.409 3602939.968

csa_tree_add_25_28          605 13603.837 1339405.062 1353008.898

mul_19_14                   262  5674.255  447626.928  453301.183

mul_20_14                   249  5647.303  420800.942  426448.246

final_adde..der_add_22_22   130  2550.018  384372.458  386922.476

sharing_mux                  25   456.713   47143.090   47599.803

sharing_mux59                25   362.531   36462.034   36824.565

RC_CG_HIER_INST0              1    41.184   27935.244   27976.429

sharing_mux60                 1    11.899     320.077     331.976

As you can see, the pipelined design’s power is much higher than the non-pipelined version. The pipelined version is more than double the power consumption.

### Pipelined:

Gate    Instances    Area        Library

------------------------------------------------

AN2D2              2     5.040    tcbn65gpluswc

AN2D4              4    17.280    tcbn65gpluswc

AN2XD1            49   105.840    tcbn65gpluswc

AN3XD1             1     2.520    tcbn65gpluswc

AO21D1            14    35.280    tcbn65gpluswc

AO22D0             1     2.880    tcbn65gpluswc

AO22D1             6    19.440    tcbn65gpluswc

AO22D2             2     7.920    tcbn65gpluswc

AOI21D1           17    36.720    tcbn65gpluswc

AOI21D2            8    25.920    tcbn65gpluswc

AOI22D0            1     2.520    tcbn65gpluswc

AOI22D1            9    22.680    tcbn65gpluswc

AOI22D2           10    43.200    tcbn65gpluswc

AOI31D2            1     4.320    tcbn65gpluswc

...

MAOI222D1         39   112.320    tcbn65gpluswc

MAOI22D1          12    34.560    tcbn65gpluswc

MOAI22D0           5    14.400    tcbn65gpluswc

MOAI22D1          93   267.840    tcbn65gpluswc

MUX2ND0            4    11.520    tcbn65gpluswc

ND2D0             20    28.800    tcbn65gpluswc

ND2D1            371   534.240    tcbn65gpluswc

ND2D2            219   551.880    tcbn65gpluswc

ND2D3              9    29.160    tcbn65gpluswc

ND2D4              8    34.560    tcbn65gpluswc

ND3D1             24    51.840    tcbn65gpluswc

ND3D2             16    51.840    tcbn65gpluswc

ND4D1              1     2.520    tcbn65gpluswc

NR2D0              4     5.760    tcbn65gpluswc

NR2D1             39    56.160    tcbn65gpluswc

NR2D2             18    45.360    tcbn65gpluswc

NR2D3              4    12.960    tcbn65gpluswc

NR2D4              1     4.320    tcbn65gpluswc

NR2XD0            67    96.480    tcbn65gpluswc

NR2XD1            69   173.880    tcbn65gpluswc

NR2XD2            35   151.200    tcbn65gpluswc

NR2XD3             2    12.240    tcbn65gpluswc

NR2XD4             2    15.840    tcbn65gpluswc

NR3D1              2     6.480    tcbn65gpluswc

OA21D1            14    35.280    tcbn65gpluswc

OA21D2             1     2.880    tcbn65gpluswc

OAI21D1           44    95.040    tcbn65gpluswc

OAI21D2           11    35.640    tcbn65gpluswc

OAI21D4            3    19.440    tcbn65gpluswc

OAI221D1           2     6.480    tcbn65gpluswc

OAI22D1           14    35.280    tcbn65gpluswc

OAI22D2            4    17.280    tcbn65gpluswc

OAI31D1            2     5.040    tcbn65gpluswc

OR2D0              1     2.160    tcbn65gpluswc

OR2D1             16    34.560    tcbn65gpluswc

OR2XD1             5    10.800    tcbn65gpluswc

XNR2D1            10    36.000    tcbn65gpluswc

XNR3D1             6    36.720    tcbn65gpluswc

XOR2D1             7    25.200    tcbn65gpluswc

XOR3D1             9    55.080    tcbn65gpluswc

------------------------------------------------

total           2550  5812.560

Type             Instances   Area   Area %

-------------------------------------------------------

sequential                          82  621.360   10.7

inverter                           506  657.720   11.3

buffer                              31   59.040    1.0

clock_gating_integrated_cell         1    6.480    0.1

logic                             1930 4467.960   76.9

-------------------------------------------------------

total                             2550 5812.560  100.0

### Non-pipelined:

Gate    Instances    Area        Library

------------------------------------------------

AN2XD1            24    51.840    tcbn65gpluswc

AO21D1             4    10.080    tcbn65gpluswc

AO221D0            2     7.200    tcbn65gpluswc

AO22D0            25    72.000    tcbn65gpluswc

AOI211XD0          1     2.520    tcbn65gpluswc

AOI21D1           12    25.920    tcbn65gpluswc

AOI22D1           54   136.080    tcbn65gpluswc

CKND0              2     2.160    tcbn65gpluswc

CKND1             37    39.960    tcbn65gpluswc

CKND2D1           53    76.320    tcbn65gpluswc

CKXOR2D1           4    14.400    tcbn65gpluswc

CMPE42D1           1    16.560    tcbn65gpluswc

DFQD1             50   342.000    tcbn65gpluswc

FCICIND1           3    14.040    tcbn65gpluswc

FICIND1            3    31.320    tcbn65gpluswc

HA1D0             11    63.360    tcbn65gpluswc

IAO21D1            2     5.040    tcbn65gpluswc

IND2D1            21    45.360    tcbn65gpluswc

INR2D1             7    15.120    tcbn65gpluswc

INR2XD0           21    45.360    tcbn65gpluswc

INVD1             50    54.000    tcbn65gpluswc

MAOI222D0         10    28.800    tcbn65gpluswc

MAOI222D1        198   570.240    tcbn65gpluswc

MAOI22D1          91   262.080    tcbn65gpluswc

MOAI22D1         141   406.080    tcbn65gpluswc

MUX2D0             1     3.240    tcbn65gpluswc

MUX2ND0            3     8.640    tcbn65gpluswc

ND2D1            187   269.280    tcbn65gpluswc

NR2D1             25    36.000    tcbn65gpluswc

NR2XD0            45    64.800    tcbn65gpluswc

OA21D1            11    27.720    tcbn65gpluswc

OA221D0            1     3.600    tcbn65gpluswc

OA22D0            20    57.600    tcbn65gpluswc

OAI211D1           1     2.520    tcbn65gpluswc

OAI21D1           14    30.240    tcbn65gpluswc

OAI221D0           4    11.520    tcbn65gpluswc

OAI22D1           42   105.840    tcbn65gpluswc

OR2D1              1     2.160    tcbn65gpluswc

OR2XD1             1     2.160    tcbn65gpluswc

XNR2D1            14    50.400    tcbn65gpluswc

XNR3D1           120   734.400    tcbn65gpluswc

XNR4D1             1     9.000    tcbn65gpluswc

XOR2D1             3    10.800    tcbn65gpluswc

XOR3D1            78   477.360    tcbn65gpluswc

------------------------------------------------

total           1399  4245.120

Type    Instances   Area   Area %

-------------------------------------

sequential        50  342.000    8.1

inverter          89   96.120    2.3

logic           1260 3807.000   89.7

-------------------------------------

total           1399 4245.120  100.0

Gate Report Comparison:

### Pipelined:

Gate    Instances    Area        Library

------------------------------------------------

AN2XD1            11    23.760    tcbn65gpluswc

AO21D1             5    12.600    tcbn65gpluswc

AO22D0            27    77.760    tcbn65gpluswc

AO22D1            29    93.960    tcbn65gpluswc

AO22D2             1     3.960    tcbn65gpluswc

AOI21D0            1     2.160    tcbn65gpluswc

AOI21D1           25    54.000    tcbn65gpluswc

AOI21D2            2     6.480    tcbn65gpluswc

AOI221D0           4    11.520    tcbn65gpluswc

AOI22D1           47   118.440    tcbn65gpluswc

...

MUX2ND0            1     2.880    tcbn65gpluswc

ND2D0              7    10.080    tcbn65gpluswc

ND2D1            169   243.360    tcbn65gpluswc

ND2D2              6    15.120    tcbn65gpluswc

ND2D3              2     6.480    tcbn65gpluswc

ND2D4              1     4.320    tcbn65gpluswc

NR2D0             10    14.400    tcbn65gpluswc

NR2D1             39    56.160    tcbn65gpluswc

NR2D3              1     3.240    tcbn65gpluswc

NR2XD0            55    79.200    tcbn65gpluswc

NR2XD1             4    10.080    tcbn65gpluswc

NR2XD2             2     8.640    tcbn65gpluswc

NR2XD3             2    12.240    tcbn65gpluswc

NR2XD4             1     7.920    tcbn65gpluswc

NR3D0              1     2.160    tcbn65gpluswc

OA21D1             9    22.680    tcbn65gpluswc

OA22D0             3     8.640    tcbn65gpluswc

OA22D1             1     3.240    tcbn65gpluswc

OAI211D0           1     2.520    tcbn65gpluswc

OAI21D0            6    12.960    tcbn65gpluswc

OAI21D1           22    47.520    tcbn65gpluswc

OAI221D0           3     8.640    tcbn65gpluswc

OAI22D1           46   115.920    tcbn65gpluswc

OAI22D2            3    12.960    tcbn65gpluswc

OAI31D0            1     2.520    tcbn65gpluswc

OR2D1              3     6.480    tcbn65gpluswc

OR2XD1             5    10.800    tcbn65gpluswc

SDFQND0            2    15.840    tcbn65gpluswc

XNR2D1            17    61.200    tcbn65gpluswc

XNR3D1            23   140.760    tcbn65gpluswc

XNR3D4             1    10.440    tcbn65gpluswc

XNR4D1             1     9.000    tcbn65gpluswc

XOR2D1             2     7.200    tcbn65gpluswc

XOR3D1            28   171.360    tcbn65gpluswc

XOR3D2             1     6.840    tcbn65gpluswc

XOR4D1             1     9.000    tcbn65gpluswc

------------------------------------------------

total           1370  3820.320

Type             Instances   Area   Area %

-------------------------------------------------------

sequential                         109  771.480   20.2

inverter                           134  163.080    4.3

clock_gating_integrated_cell         1    6.480    0.2

logic                             1126 2879.280   75.4

-------------------------------------------------------

total                             1370 3820.320  100.0

Conclusion:

The overall result of pipelining is an increase of the maximum stable clock frequency of the circuit but an increase in the area required due to the extra registers. So if area was not a concern, pipelining would be a good option to increase the clock speed limits.